The "Brown Book" for programming the IBM 1401 may be found at:

www.bitsavers.org/pdf/ibm/140x/A24-1403-5_1401_Reference_Apr62.pdf

# Memory

Every memory location has 8 bits

C	Parity bit

A	Zone bit

B	Zone Bit

8	Numeric bit

4	Numeric bit

2	Numeric bit

1	Numeric bit

M	WordMark bit

The assignment of characters to bit patterns in shown in Appendix A.

Six bits are used for characters so there are possible 64 characters.

There is no lowercase on the 1401.

The IBM 1401 could be ordered with 1,400 to 16,000 characters of memory.

Both operational 1401  machines at the CHM have 16,000 locations of memory.

All OP codes fall into one of the following formats:

Opcode

Opcode [A address]

Opcode [A address][B address]

Opcode [A address][B address][D modifier]

The IBM 1401 is a variable length machine.

Instructions can be one to 7 characters long.

The instruction length is defined by a wordmark in the opcode position and the wordmark of the following instruction.

An instruction is addressed by the opcode address.

Data fields can be from one to the size of memory long.

A data field is addressed by the right end of the data field address.

A data field is defined on the left by a wordmark at the left end of the data field.

Almost all instructions will process the data from the right end of data fields to the first wordmark found in either the A address or B address.

APPENDIX A

APPENDIX B

APPENDIX C